SAFE WORK METHOD STATEMENT (SWMS)

Western USI Pty. Ltd. – Perth Airport Advertising Installations

Combined internal terminal and external advertising installations, including installation and removal of self-adhesive vinyl (SAV) advertising on windows, walls, floors and other approved airport surfaces, and external billboard advertising-skin works. This SWMS must be read with current Perth Airport Pty Ltd (PAPL), Airport Control Centre (ACC), terminal, airside/access, permit, isolation and JCDecaux requirements. Where a current site instruction is more stringent, it applies.

| Document information | Details |
| --- | --- |
| Principal Contractor | Western USI Pty. Ltd. |
| ABN | 91 134 720 157 |
| Address | 1–5 Victory Tce, East Perth, WA 6004 |
| Activity | Installation, removal and replacement of advertising skins, self-adhesive vinyl (SAV) graphics and related advertising materials |
| Applies to | Perth Airport internal terminal and external billboard sites, plus SAV installations on windows, walls, floors and other approved surfaces throughout authorised airport areas |
| SWMS custodian | Mitchell Batterham – Managing Director |
| Issue / review date | 15-7-26 / OCTOBER 2027 |

# 1. Scope of Work

This SWMS covers notification and access, parking, unloading, movement of equipment, site establishment, public/passenger separation, internal terminal advertising installations, surface preparation and installation/removal of SAV on windows, walls, floors and other approved surfaces throughout the airport, baggage-carousel work requiring isolation, approved use of EWPs and platform ladders, external billboard access using ladders and fall protection, removal and installation of skins, pack-up, incident response and departure from Perth Airport property.

Applicability labels: “All works” applies to all advertising works; “Internal” applies to terminal, baggage-carousel and internal EWP/platform-ladder work; “External” applies to external billboard access and installation; “SAV” applies to vinyl installation/removal on approved surfaces in internal or external airport areas. Apply all relevant labels to each task.

# 2. Roles, Competency and Authorisations

| Role / requirement | Minimum requirement / responsibility |
| --- | --- |
| All installers | Read, understand and sign this SWMS; complete the pre-work assessment; comply with PAPL/ACC directions; hold required construction white card, JCDecaux and airport inductions. |
| Airport access | At least one installer must hold the access credential/ASIC required for the work area. Do not enter restricted or airside areas without current authority and escort arrangements where required. |
| Supervisor / leading hand | Competent in sign installation; confirms permits, access, isolations and controls; records changes and stops work where controls are ineffective. |
| EWP operator | Current licence/high-risk work authorisation applicable to the EWP; pre-start and logbook completed. |
| Work at height / rescue | Workers using fall-arrest equipment must be trained and competent. A suitable rescue plan and rescue equipment must be immediately available. |
| First aid | A trained first aider and suitable first-aid arrangements must be available. |

# 3. Required PPE and Safety Equipment

Long-sleeve high-visibility outerwear and long trousers suitable for PAPL requirements.

Safety footwear; steel-cap where required.

Safety glasses and task-appropriate gloves.

Sun protection for external work.

Full-body harness, compatible lanyard/double yoke and connectors where required by the approved external access method.

Barricades/concertina barriers, witches hats or bollards sufficient to establish the work area and drop zone.

First-aid kit and, where fall arrest is used, the approved rapid-response rescue kit.

Additional PPE or respiratory protection required by a product SDS or current airport/site instruction.

# 4. Perth Airport Access and Critical Controls

| Critical control | Requirement |
| --- | --- |
| Advance notification | Confirm work timing with ACC/Duty Manager before attendance. Internal aerobridge or other specially controlled works require the advance notice period directed by PAPL. JCDecaux must notify PAPL where required. |
| Arrival and departure | Report/check in with ACC or the relevant ACC/Duty Manager before work and check out when complete. Confirm restricted access times and instructions. |
| Permits | Obtain any required working-at-heights, access, hot-work, isolation or other PAPL permit before work starts. |
| Baggage systems and powered plant | BCS/Glidepath or the authorised airport service provider must implement and remove isolations. Western USI workers must not operate or remove another party’s isolation. |
| Public/passenger interface | Use hard/concertina barricading or suitable controls to isolate the work area and drop zone. Work during low passenger movement where directed and maintain a spotter/constant observation. Barricade before unloading or surface preparation. Keep accessible routes, exits, operational signage and emergency equipment clear; confirm route width with PAPL. Stop and make safe if passenger flow enters the zone. |
| Foreign object debris (FOD) | Secure tools and materials, maintain housekeeping and complete a full search before leaving. Remove all waste from site. |
| Change in conditions | Stop work if access, passenger movement, aircraft/vehicle activity, isolation, equipment, weather, lighting or other conditions invalidate the controls. |
| SAV surfaces and handover | Confirm approval for the surface, product and location before work. Do not cover safety/security signage, visibility-critical glazing or operational equipment. Keep cleaned/wet surfaces and floor graphics isolated until dry, bonded and checked for safe use. |
| Cleaning products and heat | Use products in accordance with the SDS and substrate/manufacturer instructions; provide ventilation and appropriate PPE. Any heat-assisted removal/application must be approved for the area, with required permits and ignition controls. |

# 5. Pre-Start / Site Assessment

Confirm ACC/Duty Manager notification, permitted work time, check-in and emergency contacts.

Confirm work type and applicable controls: internal, external billboard and/or SAV; identify the surface, work height and passenger/vehicle interface.

Confirm required permits, access credentials, escorts and isolations.

Inspect access route, ground/floor condition, overhead hazards, traffic and public/passenger movement.

Confirm barricade/drop-zone layout, spotter arrangements and a clear accessible passenger route; confirm the required route width with PAPL.

Inspect ladders, EWP, harnesses, lanyards, rescue equipment, tools and handling aids.

Assess loads, weather, heat, wind and lighting.

Complete the Western USI risk assessment/pre-work checklist and communicate controls to all workers.

Confirm SAV location, substrate condition and product suitability; check for loose coatings, damaged glazing and floor slip/trip hazards.

Review cleaning/removal product SDS, ventilation, PPE and any proposed heat-assisted method before use.

Confirm tools, vinyl rolls, backing paper and removed material can be controlled; protect adjacent assets and contain liquids.

# 6. Risk Rating Guide

Use the following Western USI 1A–4D matrix for all initial and residual ratings. Select the likelihood row (1–4) and consequence column (A–D); the intersecting code is the rating.

| CONSEQUENCE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | LIKELIHOOD | A Death–Catastrophic illness/injury | B Major–Extensive injuries | C Moderate Medical treatment required | D Minor No injuries |
| 1 | Almost certain Expected to occur | 1A | 1B | 1C | 1D |
| 2 | Likely Probably will occur | 2A | 2B | 2C | 2D |
| 3 | Moderate Should occur at some time | 3A | 3B | 3C | 3D |
| 4 | Unlikely Could occur at some time | 4A | 4B | 4C | 4D |
| How to use: determine the rating before controls and again after controls. Record the code in the relevant risk column. |  |  |  |  |  |

# 7. Hierarchy of Control

| Level | Application |
| --- | --- |
| 1. Elimination | Remove the hazard or avoid the hazardous work. |
| 2. Substitution | Replace the method, tool or substance with a lower-risk alternative. |
| 3. Isolation | Separate people from plant, traffic, public areas, fall zones or other hazards. |
| 4. Engineering | Use physical barriers, suitable access systems, guards or engineered measures. |
| 5. Administrative | Permits, isolations, procedures, training, spotters, timing, signage and supervision. |
| 6. PPE | Last line of defence; PPE does not replace higher-order controls. |

# 8. Detailed Task Controls

| Activity / applicability | Hazard | Initial risk | Control measures | Residual risk | Responsible |
| --- | --- | --- | --- | --- | --- |
| All works – fitness for work | Drug/alcohol impairment | 2B | Zero tolerance on PAPL land; workers must be fit for work and comply with testing requirements. | 4B | All installers |
| All works – fatigue | Reduced alertness and incident | 3B | Assess fatigue; maximum 10-hour shifts and minimum 11-hour break unless an approved system requires otherwise; complete pre-work fatigue check. | 4B | Supervisor / all |
| All works – weather | Heat, cold or wind affecting workers/materials | 3B | Check forecast and conditions; provide drinking water and breaks; secure materials; stop when heat, cold or wind makes work unsafe. | 4D | Supervisor / all |
| All works – parking/access | Vehicle or pedestrian collision | 2B | Park only in an authorised safe location; follow airport road and parking rules; use a spotter where visibility is restricted. | 4B | Driver |
| All works – unload/carry | Strain, crush injury or dropped load | 2C | Assess load; use enough people and trolleys; keep routes clear; carry long items with two people. | 4D | All installers |
| All works – establish area | Public/passenger entering work or drop zone | 3C | Install appropriate barricades and drop zone; schedule around low movement where directed; maintain a spotter/constant observation. Establish separation before unloading/preparing surfaces; maintain clear accessible routes and exits. Stop work if separation fails. | 4D | All installers |
| All works – commence | Unfamiliar task or changed conditions | 2B | All workers read/sign SWMS; complete site risk assessment and pre-work briefing; record and communicate changes before restarting. | 4B | Supervisor / all |
| All works – notify/check in | PAPL/ACC unaware of workers | 3C | Check in with ACC/Duty Manager, confirm permitted time and instructions, and check out when finished. | 4D | Leading hand |
| Internal – move EWP | Collision, crush or incorrect operation | 3B | Licensed operator only; inspect route, surface and overhead hazards; working beacon/alarm; spotter walks ahead; complete EWP pre-start/logbook. | 4D | EWP operator / spotter |
| Internal / SAV – transport EWP in lift | Lift overload or asset damage | 3C | Confirm PAPL approval, access-lift capacity and total load including EWP, people and equipment; do not exceed rated capacity. Check dimensions and floor loading; use suitable approved spreader boards where required. | 4D | EWP operator / supervisor |
| Internal – set up/use EWP | Tip-over, fall or public injury | 2C | Suitable level surface; manufacturer requirements; exclusion/drop zone; licensed operator; remain within platform; do not exceed rated capacity. | 4C | EWP operator |
| Internal – baggage-system isolation | Electrocution or unexpected movement | 2A | Isolation implemented, locked/tagged and confirmed by BCS/Glidepath or authorised provider. Do not start until isolation is verified. | 4C | Authorised isolator / installers |
| Internal / SAV – platform ladder | Fall or ladder instability | 2C | AS 1892-compliant platform ladder; pre-use inspection; firm level surface; follow manufacturer instructions; second worker to foot where necessary. Use a compliant fibreglass platform ladder for SAV tasks where appropriate to the assessed hazards; do not overreach. | 4C | Installers |
| External – access ladder | Ladder movement or fall | 2C | AS 1892-compliant ladder; firm level base; correct angle; second worker to foot; secure to suitable anchor points; maintain three points of contact. | 4C | Installers |
| External – fall exposure | Fall from height/unprotected edge | 3B | Use approved access method; full-body harness and compatible double yoke/lanyard where required; remain attached to assessed anchor; do not encroach within 3 m of an unprotected edge without effective fall protection. | 4C | Installers / supervisor |
| External – fall arrest/rescue | Suspension trauma or delayed rescue | 2B | Trained workers; site-specific rescue plan; rescue kit immediately available; suitable inspected harness and connectors; no lone work. | 4C | Supervisor / installers |
| All works – remove/install skin or SAV | Cuts, scratches and pinch points | 3C | Correct tools and PPE; coordinate work; keep hands clear of pinch points; control removed and new material. Use correct cutting tools; cut away from the body; retract/store blades and dispose of used blades in a suitable container. Protect the substrate and adjacent assets. | 4C | Installers |
| All works – tools/material overhead | Dropped objects | 3C | Exclude people below; tether tools where appropriate; secure loose materials; maintain drop zone and spotter. | 4C | Installers |
| All works – handling skins/SAV | Strains or loss of material control | 3B | Team lift; use handling aids; plan route; stop external work if wind prevents control. Control rolls and backing paper; avoid twisting and prolonged awkward postures; use team handling for large sheets. | 4D | Installers |
| All works – illness | Infectious illness affecting workers/public | 3C | Do not attend when unfit; follow current public-health, PAPL and company requirements; maintain hygiene and report relevant symptoms. | 4D | All installers |
| SAV – prepare/clean surface | Chemical exposure, slips or asset damage | 3C | Confirm substrate and product compatibility; read SDS; use task-appropriate gloves/eye protection and respiratory protection where required; ventilate; do not mix chemicals. Contain liquids and isolate wet surfaces until dry. | 4D | Installers |
| SAV – remove existing vinyl | Cuts, strain or loss of material control | 3C | Use suitable tools and a controlled removal method; keep hands clear and coordinate team work; control removed vinyl. Use heat only where approved, in accordance with equipment/product instructions and required airport permits; keep hot tools clear of people and ignition hazards. | 4C | Installers / supervisor |
| SAV – apply graphics | Passenger struck or exposed to work | 3B | Maintain barricades and constant observation; exclude passengers below and adjacent to the task. Control large sheets and backing paper; stop and make safe if passenger flow enters the work zone. | 4D | Installers / spotter |
| SAV – elevated work | Dropped object or fall from access equipment | 2B | Secure tools/materials; maintain exclusion/drop zone. Use approved EWP/platform ladder on firm level surface in accordance with manufacturer instructions; follow applicable airport access and fall-protection controls. | 4D | Installers |
| SAV – floor graphics | Slip/trip hazard from liquid or lifting edges | 3C | Use an approved floor-graphic system suitable for the intended surface and pedestrian use. Barricade throughout preparation/application; remove liquids, backing paper and offcuts. Check bonding, edges and surface condition before reopening; isolate and rectify lifting/damaged graphics. | 4D | Installers / leading hand |
| Public/passenger interface | Aggressive or abusive person | 3C | Use authorised low-movement work windows where practicable; remain calm and respectful; withdraw and contact airport staff/security if behaviour escalates. | 4D | All installers |
| All works – pack up | Manual handling strain | 2C | Use team lifts and handling aids; plan route; avoid twisting; break down loads where practicable. | 4D | All installers |
| SAV – de-installation | Hazards during reverse process | 2A | Repeat applicable airport access, isolation, barricading, ladder/EWP, cutting, chemical, manual-handling and pack-up controls during removal. Inspect and leave the surface and passenger route safe. | 3C | Supervisor / all |
| All works – pack up/FOD | Tools, rubbish or equipment left behind | 3C | Full work-area search; account for tools/materials; remove waste; leave routes and airport assets clear. Contain SAV backing paper, offcuts, removed vinyl and used blades; prevent loose material reaching aircraft/vehicle areas. Keep barricades until all hazards are removed. | 4C | All installers |
| All works – load vehicle | Unsecured equipment/material | 3B | Pack and secure load; close doors/tailgates; final check before moving vehicle. | 4D | Driver / installers |
| All works – emergency | Delayed response or recurrence | 3B | Stop, make safe, first aid/000 as required; notify ACC/PAPL and supervisor; report incidents and near misses under current company/client process. | 4D | All installers |
| All works – leave site | Vehicle collision or incomplete checkout | 2A | Check out with ACC/Duty Manager; confirm isolations are handed back by authorised party; take care re-entering traffic. | 3C | Driver / leading hand |

# 9. Plant, Equipment and Safety Checks

| Equipment | Pre-use / maintenance requirement |
| --- | --- |
| Motor vehicles | Roadworthy, licensed and inspected; load restraints suitable. |
| EWP | Current inspection/service; pre-start and logbook; alarms and emergency lowering functional. |
| Ladders | Correct type, AS 1892 compliant, inspected and undamaged. |
| Harnesses, lanyards and connectors | Current inspection/certification; compatible and used only by trained workers. |
| Rescue equipment | Complete, inspected, accessible and suitable for the rescue plan. |
| Hand/battery tools | Fit for purpose, inspected and secured where dropped-object risk exists. |
| Barricades / cones / bollards | Sufficient, stable and maintained throughout the work. |
| Trolleys / handling aids | Suitable capacity, stable and inspected. |
| SAV tools / cleaning equipment | Inspect knives, scrapers and squeegees; retract/store blades; provide suitable blade disposal, labelled cleaning products and current SDS. Inspect electrical/heat tools before authorised use. |
| Spreader boards / surface protection | Suitable, stable and inspected; verify loading and PAPL approval before use with an EWP or access lift. |

# 10. Emergency and Incident Response

For immediate danger or serious injury call 000, notify ACC/PAPL and follow airport emergency directions.

Stop work, make the area safe and provide first aid within training and competence.

For a fall-arrest event, implement the site rescue plan immediately; do not rely solely on emergency services for initial recovery.

Do not approach or operate isolated electrical/mechanical equipment; contact the authorised isolation provider.

Report all incidents, hazards and near misses to the supervisor and under current PAPL/JCDecaux/Western USI procedures.

# 11. On-Site Changes / Dynamic SWMS Review

STOP and review this SWMS when access, work area, passenger movement, traffic, isolation, weather, equipment, personnel, method or scope changes. Record the change, revised controls and risk rating; brief affected workers and obtain initials before work recommences.

| Date/time | Change / new hazard | Risk before | Additional control / revised method | Risk after | Approved / briefed by | Worker initials |
| --- | --- | --- | --- | --- | --- | --- |

# 12. SWMS Consultation and Sign-Off

Every member of the work crew must read and understand this SWMS, have the opportunity to ask questions and sign below before commencing work. Revised controls must be explained and workers must re-sign where required.

| Name | Company / role | Signature | Date |
| --- | --- | --- | --- |

# 13. Document Review and Approval

| Prepared / reviewed by | Position | Signature | Date |
| --- | --- | --- | --- |
| Mitchell Batterham | Managing Director / SWMS Custodian |  | 10/08/2026 |
|  | PAPL / client representative (where required) |  |  |

Review trigger: annual review, after an incident or near miss, after significant change to work method/equipment/site requirements, or when required by PAPL or the client.